Εισαγωγικά παραδείγματα VHDL

Entity, Architecture

Περιγραφή βασικών πυλών με VHDL

Υλοποίηση Testbenches

Γνωριμία με το Sonata της Simili EDA

Basic digital logic gates not, and, or, nand, nor, xor, xnor.

Not

--import std_logic from the IEEE library

library ieee;

use ieee.std_logic_1164.all;

--ENTITY DECLARATION: name, inputs, outputs

entity notGate is

port( inPort  : in std_logic;

outPort : out std_logic);

end notGate;

--FUNCTIONAL DESCRIPTION: how the Inverter works

architecture func of notGate is

begin

outPort <= not inPort;

end func;

And

--import std_logic from the IEEE library

library ieee;

use ieee.std_logic_1164.all;

--ENTITY DECLARATION: name, inputs, outputs

entity andGate is

port( A, B : in std_logic;

F : out std_logic);

end andGate;

--FUNCTIONAL DESCRIPTION: how the AND Gate works

architecture func of andGate is

begin

F <= A and B;

end func;

Or

--import std_logic from the IEEE library

library ieee;

use ieee.std_logic_1164.all;

--ENTITY DECLARATION: name, inputs, outputs

entity orGate is

port( A, B : in std_logic;

F : out std_logic);

end orGate;

--FUNCTIONAL DESCRIPTION: how the OR Gate works

architecture func of orGate is

begin

F <= A or B;

end func;

Nand

--import std_logic from the IEEE library

library ieee;

use ieee.std_logic_1164.all;

--ENTITY DECLARATION: name, inputs, outputs

entity nandGate is

port( A, B : in std_logic;

F : out std_logic);

end nandGate;

--FUNCTIONAL DESCRIPTION: how the NAND Gate works

architecture func of nandGate is

begin

F <= A nand B;

end func;

Nor

--import std_logic from the IEEE library

library ieee;

use ieee.std_logic_1164.all;

--ENTITY DECLARATION: name, inputs, outputs

entity norGate is

port( A, B : in std_logic;

F : out std_logic);

end norGate;

--FUNCTIONAL DESCRIPTION: how the NOR Gate works

architecture func of norGate is

begin

F <= A nor B;

end func;

Xor - Xnor

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity xor_xnor_top is

Port ( A1 : in  STD_LOGIC;      -- XOR gate input 1

A2 : in  STD_LOGIC;      -- XOR gate input 2

X1 : out  STD_LOGIC;     -- XOR gate output

B1 : in  STD_LOGIC;      -- XNOR gate input 1

B2 : in  STD_LOGIC;      -- XNOR gate input 2

Y1 : out  STD_LOGIC);    -- XNOR gate output

end xor_xnor_top;

architecture Behavioral of xor_xnor_top is

begin

X1 <= A1 xor A2;  -- 2 input exclusive-OR gate

Y1 <= B1 xnor B2;  -- 2 input exclusive-NOR gate

end Behavioral;

Test-bench creation

not simulation

--import std_logic from the IEEE library

library ieee;

use ieee.std_logic_1164.all;

--ENTITY DECLARATION: no inputs, no outputs

entity notGate_tb is

end notGate_tb;

-- Describe how to test the inverter

architecture tb of notGate_tb is

--pass notGate entity to the testbench as component

component notGate is

port( inPort  : in std_logic;

outPort  : out std_logic);

end component;

signal  goIn, goOut : std_logic;

begin

--map the testbench signals to the ports of the notGate

mapping: notGate port map( goIn, goOut);

process

begin

--TEST 1

goIn <= '0';

wait for 15 ns;

--TEST 2

goIn <= '1';

wait for 15 ns;

end process;

end tb;

and simulation

--import std_logic from the IEEE library

library ieee;

use ieee.std_logic_1164.all;

--ENTITY DECLARATION: no inputs, no outputs

entity andGate_tb is

end andGate_tb;

-- Describe how to test the AND Gate

architecture tb of andGate_tb is

--pass andGate entity to the testbench as component

component andGate is

port( A, B : in std_logic;

F : out std_logic);

end component;

signal  inA, inB, outF : std_logic;

begin

--map the testbench signals to the ports of the andGate

mapping: andGate port map(inA, inB, outF);

process

begin

--TEST 1

inA <= '0';

inB <= '0';

wait for 15 ns;

--TEST 2

inA <= '0';

inB <= '1';

wait for 15 ns;

--TEST 3

inA <= '1';

inB <= '1';

wait for 15 ns;

end process;

end tb;

or simulation

--import std_logic from the IEEE library

library ieee;

use ieee.std_logic_1164.all;

--ENTITY DECLARATION: no inputs, no outputs

entity orGate_tb is

end orGate_tb;

-- Describe how to test the OR Gate

architecture tb of orGate_tb is

--pass orGate entity to the testbench as component

component orGate is

port( A, B : in std_logic;

F : out std_logic);

end component;

signal  inA, inB, outF : std_logic;

begin

--map the testbench signals to the ports of the orGate

mapping: orGate port map(inA, inB, outF);

process

begin

--TEST 1

inA <= '0';

inB <= '0';

wait for 15 ns;

--TEST 2

inA <= '0';

inB <= '1';

wait for 15 ns;

--TEST 3

inA <= '1';

inB <= '1';

wait for 15 ns;

end process;

end tb;

nand simulation

--import std_logic from the IEEE library

library ieee;

use ieee.std_logic_1164.all;

--ENTITY DECLARATION: no inputs, no outputs

entity nandGate_tb is

end nandGate_tb;

-- Describe how to test the NAND Gate

architecture tb of nandGate_tb is

--pass nandGate entity to the testbench as component

component nandGate is

port( A, B : in std_logic;

F : out std_logic);

end component;

signal  inA, inB, outF : std_logic;

begin

--map the testbench signals to the ports of the nandGate

mapping: nandGate port map(inA, inB, outF);

process

begin

--TEST 1

inA <= '0';

inB <= '0';

wait for 15 ns;

--TEST 2

inA <= '0';

inB <= '1';

wait for 15 ns;

--TEST 3

inA <= '1';

inB <= '1';

wait for 15 ns;

end process;

end tb;

nor simulation

--import std_logic from the IEEE library

library ieee;

use ieee.std_logic_1164.all;

--ENTITY DECLARATION: no inputs, no outputs

entity norGate_tb is

end norGate_tb;

-- Describe how to test the NOR Gate

architecture tb of norGate_tb is

--pass norGate entity to the testbench as component

component norGate is

port( A, B : in std_logic;

F : out std_logic);

end component;

signal  inA, inB, outF : std_logic;

begin

--map the testbench signals to the ports of the norGate

mapping: norGate port map(inA, inB, outF);

process

begin

--TEST 1

inA <= '0';

inB <= '0';

wait for 15 ns;

--TEST 2

inA <= '0';

inB <= '1';

wait for 15 ns;

--TEST 3

inA <= '1';

inB <= '1';

wait for 15 ns;

end process;

end tb;

Exercise

Create and test the testbench for xor/xnor simulation.