Decoders

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

ENTITY decode1 IS

PORT(

d1, d0 : IN STD_LOGIC;

y0, y1, y2, y3 : OUT STD_LOGIC);

END decode1;

ARCHITECTURE decoder1 OF decode1 IS

BEGIN

y0 <= (not d1) and (not d0);

y1 <= (not d1) and ( d0);

y2 <= ( d1) and (not d0);

y3 <= ( d1) and ( d0);

END decoder1;

Use vectors:

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

ENTITY decode2 IS

PORT (

d : IN STD_LOGIC_VECTOR (1 downto 0);

y : OUT STD_LOGIC_VECTOR (3 downto 0));

END decode2;

ARCHITECTURE decoder2 OF decode2 IS

BEGIN

y(0) <= (not d(1)) and (not d(0));

y(1) <= (not d(1)) and ( d(0));

y(2) <= ( d(1)) and (not d(0));

y(3) <= ( d(1)) and ( d(0));

END decoder2;

Use With

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

ENTITY decode2a IS

PORT (

d : IN STD_LOGIC_VECTOR (1 downto 0);

y : OUT STD_LOGIC_VECTOR (3 downto 0));

END decode2a;

ARCHITECTURE decoder OF decode2a IS

BEGIN

—— based on binary value of d

WITH d SELECT

y <= “0001” WHEN “00”,

“0010” WHEN “01”,

“0100” WHEN “10”,

“1000” WHEN “11”,

“0000” WHEN others;

END decoder;

Comparators

—— compare2.vhd

ENTITY compare2 IS

PORT(

a, b : IN BIT_VECTOR (1 downto 0);

agtb, aeqb, altb : OUT BIT);

END compare2;

ARCHITECTURE a OF compare2 IS

BEGIN

altb <= (not (a(1)) and b(1)) or ((not (a(1) xor b(1))) and (not (a(0)) and b(0)));

aeqb <= (not (a(1) xor b(1))) and (not (a(0) xor b(0)));

agtb <= (a(1) and not (b(1))) or ((not (a(1) xor b(1))) and (a(0) and not (b(0))));

END a;

four-bit comparator

LIBRARY ieee ;

USE ieee.std logic 1164.all ;

USE ieee.std logic unsigned.all ;

ENTITY compare IS

PORT ( A, B : IN STD LOGIC VECTOR(3 DOWNTO 0) ;

AeqB, AgtB, AltB : OUT STD LOGIC ) ;

END compare ;

ARCHITECTURE Behavior OF compare IS

BEGIN

AeqB <= ’1’ WHEN A=  B ELSE ’0’ ;

AgtB <= ’1’ WHEN A > B ELSE ’0’ ;

AltB <= ’1’ WHEN A < B ELSE ’0’ ;

END Behavior ;